NovaPay Hybrid Network — Capstone Project

A fictional mid-market payments processor running hybrid infrastructure: an on-premises office/data center connected to AWS-hosted payment processing via Site-to-Site VPN.

Built to demonstrate distributed systems design across a full hybrid network stack.

Architecture Overview

On-Premises (GNS3)

- VyOS R1 — inter-VLAN routing, NAT, VPN termination

- 2x Ethernet switches — VLAN segmentation, 802.1q trunking

- 10 endpoints across 4 VLANs (Payments, IT, Operations, Management)

AWS Cloud (us-east-1)

- VPC: 10.0.0.0/16

- ALB across two public subnets, EC2 in private subnets, RDS in isolated subnets

- Site-to-Site IPSec VPN via Virtual Private Gateway

If You Want to Spin This Up

⚠️ This environment was built in a session-based lab. Several values are environment-specific and must be updated before deploying.

1. Update Terraform Variables

In variables.tf, replace the following with your own values:

- VPC ID

- Subnet IDs (public and private)

- Key pair name

In main.tf, add your AWS credentials:

- access_key

- secret_key

Then run:

terraform init

terraform apply

2. Open the GNS3 Project

- Open GNS3

- File → Open Project → select the .gns3 file from the repo folder

- Start all devices

3. Configure the VyOS Router

In the VyOS console inside GNS3, Run these commands:

configure

set interfaces ethernet eth0 address 192.168.100.2/24

set protocols static route 0.0.0.0/0 next-hop 192.168.100.1

set service ssh port 22

commit

save

4. Load the Config File

From your host machine terminal, SCP the config into the router:

scp vyos-config.boot vyos@192.168.100.2:/config/config.boot

Then back in the VyOS console, Run these commands:

configure

load /config/config.boot

commit

save

5. Assign IPs to Workstations

VPC configurations don’t last on reboot of the environment, On each VPCs workstation, run:

ip dhcp

This pulls a DHCP lease from VyOS per the VLAN pool.

6. Reconfigure the Ubuntu Server Static IP

The Ubuntu server (UbuntuServer-1, VLAN 20) loses its netplan config on lab restart and must be set manually.

In the Ubuntu console inside GNS3:

Check for conflicting netplan files and remove the cloud-init one:

ls /etc/netplan/

sudo rm /etc/netplan/50-cloud-init.yaml

Disable cloud-init network management so it doesn't override your config on next boot:

sudo nano /etc/cloud/cloud.cfg.d/99-disable-network-config.cfg

Add this line: Edit the static IP config:

sudo nano /etc/netplan/00-installer-config.yaml

Paste:

network:

version: 2

ethernets:

ens3:

dhcp4: false

addresses:

- 192.168.20.10/24

routes:

- to: default

via: 192.168.20.1

nameservers:

addresses:

- 8.8.8.8

Set permissions and apply:

sudo chmod 600 /etc/netplan/00-installer-config.yaml

sudo netplan apply

Verify with ip a — you should see 192.168.20.10/24 on ens3.

Note on the VPN Tunnel

The Site-to-Site VPN tunnel is not expected to be fully reproduced — it requires live AWS environment values that change each session.

If you do want to bring it up:

- After terraform apply, grab the pre-shared key and tunnel outside IP from the AWS VPN connection in the console

- Update those values in the VyOS config on R1

- The tunnel should negotiate and come up — some troubleshooting may be needed depending on your environment

Otherwise, TC1–TC6 are fully verifiable with these instructions.

Test Cases

| # | Test Case | Status |
| --- | --- | --- |
| TC1 | Local Network Segmentation | ✅ |
| TC2 | DHCP + Static Addressing | ✅ |
| TC3 | Inter-VLAN Access Controls | ✅ |
| TC4 | Cloud Network Segmentation | ✅ |
| TC5 | Cloud Addressing | ✅ |
| TC6 | Cloud Access Controls | ✅ |
| TC7 | VPN Bridge | ✅ |
| TC8 | Hybrid Network Security | ✅ |